Invitation for Bids (IFB)

| Country: | REPUBLIC GEORGIA |
| --- | --- |
| Project: | Rehabilitation of Communal Infrastructure of New Batumi (BAT V) – Sewer Pumping Station B and Closing of Deep Sewer Interconnections |
| Tender No.: | BAT V - WC 2 |
| Procurement No.: | 512246 |
| Date: | 17th April 2026 |

- The Municipality of Batumi has received financing from KfW toward the cost of the Project “Rehabilitation of Communal Infrastructure of New Batumi (BAT – V)” and intends to apply part of the proceeds toward payments under the contract for “Pumping Station B and Closing of Deep Sewer Interconnections”.

- The Municipality of Batumi now invites sealed Bids from eligible bidders for implementation of the following works, consisting of 2 LOTs which must be implemented in parallel under the FIDIC Pink Book conditions of Contract, 2010.

  - LOT 1: Construction of a new sewer Pumping Station PSB

  - Construction of a new sewer PS B, to be implemented at the site of the existing old PS and to be constructed fully in parallel with the operating old PS B, with a minimum of operational interruptions. The new PS to safely convey wastewater and unavoidable stormwater and groundwater infiltrations from sewer system B to Adlia WWTP. The new PS is designed to latest state-of-the-art standards and technology. Works to include:

    - Deep excavations up to 10 m, with a high ground water table. Sheet piling and grout soil conditioning measures are required.

    - Supply and install 6 dry installed pumps, 3 duty and 3 stand by, each pump with capacity of 312 l/s @ 45 m.

    - Supply and install two Multi-Rake Bar Screens, each for a nominal capacity of 935 l/s but able to operate for limited period also for 1,300 l/s.

    - Supply and install 4 number 15m3 stainless steel surge vessels, to prevent water hammer in the pumping main.

    - Air abstraction and odor treatment.

    - Standby generator, supply and installation of a comprehensive PLC based automation control system for operation of the PS in automatic, semi-automatic, manual and emergency modes.

    - Optimisation of PS performance during commissioning, including the control system and pump operation.

  - LOT 2: Closing of deep city sewer interconnections

  - Closing of two (2) interconnection points between the old sewer collectors and the new sewer system in depths of up to 6.5 m. Works to include:

    - Laying 31 m of new DN500 polypropylene pipes.

    - Construction of new manhole with a depth of 4.50 m.

    - Backfilling old manholes and collectors with concrete slurry, cutting of old steel pipes.

    - Pit securing by sheet piling or equivalent.

- The contract will allow for a Construction Period of 580 days and a Defect Notification Period of 360 days.

- Bidding will be conducted through the International Competitive Bidding (ICB) Single-Stage Selection procedures as specified in the KfW Guidelines for Procurement of Goods, Works and associated Services in Financial Cooperation with Partner Countries (January 2019, 2nd update in November 2023). The Guidelines can be downloaded from www.kfw-entwicklungsbank.de/International-financing/KfW-Development-Bank/Publications/Guidelines. Application is open to all eligible bidders as defined in the KfW Guidelines.

- A complete set of bidding documents in electronic form will be made available upon submission of an application to the address below. The documents are free of charge. Bidders must confirm receipt of the electronic bidding documents by letter or e-mail. Only confirmed electronic versions are accepted as bidding documents issued by the Employer!

- A Pre-Bid Meeting and Site Visit will take place at the address specified below on Thursday 21st of May 2026 starting at 10:00hrs Batumi time.

- Bids must be delivered to the address indicated in the clause ITB 22.1 of the tender documents on or before 11:00 a.m Batumi Time on Thursday 11th of June 2026. Late bids will be rejected. The Qualification Document opening (first public session) shall take place in the presence of the bidders’ designated representatives at the address below on Thursday 11th of June 2026 at 11:30hrs Batumi time.

- All bids must be accompanied by a bid security of EURO 100.000,00 (hundred thousand Euro).

- The address referred to above is:

Batumi Municipality City Hall

Attn: Mrs. Maia Zoidze

Head of the Investment and Grant Programs Management Division 
of the Economic Development and Municipal Organizations Management Service

Luka Asatiani Street No. 25

City: Batumi

ZIP Code: 6010

Country: Georgia

Telephone: +995 422 27 26 27

Electronic mail address:

batumiproject@gmail.com, pea@batumi.ge